|  | #### Job Safety Analysis |  |  |
| --- | --- | --- | --- |
|  | Facilities Services |  |  |
|  | ## Operating a Wood Chipper |  |  |
|  | Task | Hazards | Controls |
|  | - Secure chipper to towing vehicle. | Pinching fingers, hands, legs, and feet | Avoid pinch points when connecting chipper hitch, safety chain, and brake lights to towing vehicle. |
|  | 2. Prepare work area. | Traffic hazards, injuries to bystanders and co-workers. | Set out warning signs or traffic cones. Leave room on curbside of chipper for workers to maneuver. Clear feeder table of all objects, position discharge chute to direct chips in desired direction. |
|  | 3. Operate chipper. | Flying debris entering eyes or injuring head | Wear safety glasses or face shield and hard hat; do not feed foreign material such as stones, nails, or sweepings into chipper. |
|  |  | Cutting injuries to hands, body parts | Do not wear loose-fitting clothing or gauntlet-type gloves. Do not place hands, arms, feet, legs, or any body parts past the front plane of the feeder table when the chipper is in operation. Feed chipper from the curb side of the center line and immediately pivot away from the feeder table when material is being pulled into the chipper. Use large limbs to push small pieces through blades. If chute becomes clogged or chipper requires maintenance, fueling, or lubrication, turn off engine and remove key before performing these operations. Do not operate chipper when panels or chute are removed. |
|  |  | Hearing loss | Wear ear plugs or ear muffs |
|  | Required Training: 1. Operation of wood chipper | Required Personal Protective Equipment (PPE) - Hard hat - Ear plugs - Safety glasses - Face shield - Loose-fitting, non-gauntlet type gloves - Work boots |  |