| X X | JOB TITLE: Lawn Mowing Page 1 of 2 | JSA No. 200-64-001 | DATE: 6/19/03 | NEW x REVISED |
| --- | --- | --- | --- | --- |
| JOB SAFETY ANALYSIS | TITLE OF PERSON WHO DOES JOB: Public Works Tech | SUPERVISOR: Frank Urquiza | ANALYSIS PERFOMED BY: Crew/ Frank Urquiza |  |
| ORGANIZATION: MCDOT | LOCATION: Sun City Maintenance Area | DEPARTMENT: 95-3 | REVIEWED BY: |  |
| SEQUENCE OF BASIC JOB STEPS | POTENTIAL HAZARDS | RECOMMENDED ACTION OR PROCEDURE |  |  |
| Find Location | N/A | N/A |  |  |
| Checking area before starting mower | Bottles, glass, cans branches hidden in taller grass. Signs and markers in area to be mowed Co-workers in same area | Do a walk around, use spotter, be aware of your surroundings/traffic Ensure you do not block any traffic signs and park clear of vehicle traffic Wear Safety vest or Orange shirt/jacket, place traffic cones appropriate for traffic flow and clear area informing co-workers you are going to start mowing Wear Proper Personal Protective Equipment: Hard hat, leather gloves, Steel-toed boots, sun glasses |  |  |
| Starting lawnmower | Fluid spills Sparks from bad switches Sharp or loose blades Loose wheels Unstable seat or handle | Maintain spill by spreading and covering with sand. Check all switches to ensure not loose or missing Use proper procedure for filling gas tanks boots, sun glasses Check to ensure blades are tightened and in good condition Inspect wheels to make sure they are not loose Make sure handle and /or seat is secure. Be aware of your surroundings Wear protective clothing, safety Glasses, steel toed shoes, and hard hat to prevent any injury while mowing . |  |  |
| Completing the Task | Burns from hot pipes or switches Injury while cleaning and loading Mower. Back strain while loading | Don’t start post-use inspection until unit has had time to cool off Clean with the proper tools and supplies Ask for and maintain assistance in loading mower, use proper lifting techniques. |  |  |