|  | Injury & Illness Prevention Program Job Safety Analysis Form |
| --- | --- |

| Picture of task/equipment: |  | Task: |  | Using Hypodermic Needles |
| --- | --- | --- | --- | --- |
|  |  | Name of Shop or Dept: |  |  |
|  |  | Job Title(s): |  |  |
|  |  | Analyzed by: |  |  |
|  |  | Date: |  |  |
| Required PPE: |  |  |  |  |
| - Lab Coat - Closed Toe Shoes - Gloves |  |  |  |  |
| Required/Recommended Trainings: |  |  |  |  |
| - Bruin Safety Training - Lab Safety Training |  |  |  |  |
| TASK | HAZARDS |  | CONTROLS |  |
| 1.Attach needle to syringe |  |  |  |  |
| 2.Remove protective sleeve from needle |  |  | 2a1. Hold needle directed away from your body and carefully remove protective sleeve. Carefully insert the needle into sterile port of container. Draw desired volume into syringe, remove from container, and insert into new container. 2a2. Dispose of needle with attached syringe in proper sharps container as soon as possible. If the needle must be removed from a syringe, use a needle/sharps container with the "integral needle removing device" in the lid. Do not cut or bend needles prior to disposal. 2a3. Do not replace protective sleeve on used needles unless absolutely necessary. If needles must be recapped, use a one-handed technique by sliding the needle into a sleeve that is either sitting in a rack or is lying on a flat surface, that is to say, do not hold the sleeve while reinserting the needle. |  |
| 3.Carry out specific task(eg. Transferring sterile growth factors,chemical solutions,blood products,etc. from sterile container | 3a. Needle may puncture your skin |  | 2a1. Hold needle directed away from your body and carefully remove protective sleeve. Carefully insert the needle into sterile port of container. Draw desired volume into syringe, remove from container, and insert into new container. 2a2. Dispose of needle with attached syringe in proper sharps container as soon as possible. If the needle must be removed from a syringe, use a needle/sharps container with the "integral needle removing device" in the lid. Do not cut or bend needles prior to disposal. 2a3. Do not replace protective sleeve on used needles unless absolutely necessary. If needles must be recapped, use a one-handed technique by sliding the needle into a sleeve that is either sitting in a rack or is lying on a flat surface, that is to say, do not hold the sleeve while reinserting the needle. |  |
| 4.Dispose of Hypodermic Needle | 4a.Needle may puncture other personnel |  | 4a1.Always dispose of needles in appropriate approved "Sharps" or "Biohazard Sharps" container. Biohazard containers may not be orange. |  |