|  | Injury & Illness Prevention Program Job Safety Analysis Form |
| --- | --- |

| Picture of task/equipment: |  | Task: |  | Vehicle Battery Removal/Installation |
| --- | --- | --- | --- | --- |
|  |  | Name of Shop or Dept: |  |  |
|  |  | Job Title(s): |  |  |
|  |  | Analyzed by: |  |  |
|  |  | Date: |  |  |
| Required PPE: |  |  |  |  |
| Work Gloves Safety Glasses Closed Toe Shoes |  |  |  |  |
| Required/Recommended Trainings: |  |  |  |  |
| - Materials Handling - Safe Lifting/Back Safety |  |  |  |  |
| TASK | HAZARDS |  | CONTROLS |  |
| 1. Restrain Wheels | Cart can roll away |  | - Chock Wheels |  |
| 2. Remove Obstructions | Strain injuries due to awkward movements when removing batteries |  | - Remove all braces, cables & obstructions blocking battery removal - DO NOT remove battery without first removing braces, cables and other obstructions |  |
| 3. Put battery lift into position | Trips/Falls |  | - Move battery lift into position near vehicle - Slide feet of battery lift under vehicle - Watch your step - Move crane into the vehicle |  |
| 4. Attach battery to battery lift crane | Dropped battery |  | - Inspect battery handle for wear - If worn or damaged, attach a battery terminal grabber (see second picture) - Attach crane hook to lift the battery |  |
| 5. Remove Batteries with battery lift | Strain injuries due to awkward movements when removing batteries |  | - Use IN/OUT switch on battery lift to operate battery lift. OUT position will cause crane to start lifting the battery. - Carefully lift battery out of vehicle - Guide crane with your hands to direct battery out of compartment - DO NOT lift the battery manually - Keep face away from lift crane hook - Move crane arm out of vehicle to desired position - IN position of switch will gradually lower the battery to the ground - Unhook battery handle from crane |  |