| Pre-use Inspection Checklist for Hand-operated Hoists Includes wire rope, chain, web-strap, manual-lever-operated (come-a-longs) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Facilities CRN#: PC#: Hoist Type: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Crane Custodian: Bldg: Room: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Requirements - Operators must perform a pre-use inspection (once per shift) for the items listed below. - Any equipment found to be unsatisfactory must be removed from service. - Operators must notify the equipment custodian, who will submit a Facilities service request to the crane inspector (call x8901). Continued use may be permitted if authorized by the crane inspector in notes/comments. - Custodians must keep completed checklists on file for a minimum of 5 years. |  | Inspected by: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Date: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # | Equipment Description and Inspection Criteria | = Satisfactory, U = Unsatisfactory, N = Not Applicable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Operating mechanism – Check for proper operation and adjusts, no unusual sounds or noise due to chain binding or bearing squeal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Hooks – Check for distortion such as bending, twisting, or increased throat opening; wear, cracks, nicks or gouges; hook attachment and securing means |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | Hook latches – If provided, check for latch engagement, damaged or malfunctioning latch, check self-locking hooks for proper operation and locking |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4-a | Load chain– (Welded Link) Check for wear, stretching, gouges, nicks, weld splatter, corrosion and distorted links; (Roller Chain) in addition to the preceding, check that: rollers run freely; side plates are not spread open; there is no pitting or discoloration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4-b | Wire rope – Check for broken, bent, kinked, distorted, birdcaging, crushed or corroded wire |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4-c | Web strap – Check for melting or charring, chemical burns, weld splatter, broken stitches, cuts or tears, damage to eyes or fittings, abrasive wear, knots |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | Reeving – Check for proper reeving (chain, rope, or strap type) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | Hoist leaver – Check lever for bends, cracks, or other damage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 | Hoist support – Check for damage to hoist support |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes/comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |