# HAZARD ASSESSMENT

| Location: |  |  | Conducted By: |  | Date: |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tasks performed at this location: |  |  |  |  |  |  |

Directions: Review the tasks at the worksite to determine if any of these hazards exist.  Place an “x” in the appropriate box.  If any of the indicated hazards are present, describe the hazard, eliminate it or identify the appropriate PPE to be used.

| OVERHEAD HAZARDS |  | Yes | No | Recommended PPE | If yes, describe hazard details |
| --- | --- | --- | --- | --- | --- |
| 1 | Suspended/elevated loads, beams, or objects that could fall or strike head | x |  | Hard hat, ANSI Z89, Class G, E or C | Hazard during general maintenance activities. Operating overhead cranes. Hard hat required. |
| 2 | Flying objects that could strike head | x |  | Hard hat, ANSI Z89, Class G, E or C | Hazard during general maintenance activities. Operating overhead cranes. Hard hat required. |
| 3 | Energized wires or equipment that could strike head | x |  | Hard hat, ANZI Z89, Class G or E (dependent on potential voltage) | Hazards during troubleshooting live electrical and performing live electrical work. Arc flash protection required. |
| 4 | Sharp objects or corners at head level | x |  | Hard hat, ANSI Z89, Class G, E or C | Hazards during maintenance on equipment. Hard hat required. |
| EYE HAZARDS |  | Yes | No | Recommended PPE | If yes, describe hazard details |
| 5 | Chemical splashes or irritating mists | x |  | See Supplemental Information A for additional information | Working in the chemical areas and using cleaning agents. Safety glasses and face shield required. |
| 6 | Excessive dust | x |  | Safety glasses or goggles | Excessive dust created when sweeping floors and working overhead. Sealed safety glasses required. |
| 7 | Smoke and/or fumes |  | x | Safety goggles |  |
| 8 | Welding operations | x |  | Welding goggles; See Supplemental Information A and B for additional information | Hazard when welding. Welding shield required. |
| 9 | Lasers/optical radiation |  | x | Have URS HSE Representative assist you in proper selection |  |
| 10 | Projectiles | x |  | Safety glasses or goggles plus face shield | Hazards during general maintenance activities. Safety glasses required. |
| 11 | Sawing, cutting, chipping, and/or grinding | x |  | Safety glasses or goggles plus face shield; See Supplemental Information A for additional information | Grinders used for maintenance activities. Face shield required. |
| FACE HAZARDS |  | Yes | No | Recommended PPE | If yes, describe hazard details |
| 12 | Chemical splashes or irritating mists | x |  | Safety goggles; See Supplemental Information A for more information; add face shield if irritating or corrosive | Hazards working in the chemical areas and using cleaning agents. Safety glasses and face shield required. |
| 13 | Welding operations | x |  | Welding goggles or welding helmet; see Supplemental Information A and B for additional information | Hazard when welding. Welding shield required. |
| 14 | Projectiles |  | x | Safety glasses or goggles plus face shield |  |

| HAND HAZARDS |  | Yes | No | Recommended PPE | If yes, describe hazard details |
| --- | --- | --- | --- | --- | --- |
| 15 | Chemical exposure | x |  | Use chemical-resistant gloves specific to hazard; consult MSDS, chemical hazard guide, or HSE Representative | Hazards when handling general paints and oils, cleaning agents and any other chemical. Chemical protective gloves required. Nitrile for paints, oils and cleaning chemicals. Neoprene required for work in chemicals. |
| 16 | Sharp edges, splinters, etc. | x |  | Leather or Kevlar gloves | Hazards exist throughout the plant. General use gloves required with a minimum cut resistance of 2. |
| 17 | Temperature extremes – heat | x |  | Leather gloves, welder’s gloves, hot mill Gloves | Welding operations. Welding gloves required. |
| 18 | Temperature extremes – cold |  | x | Insulated gloves |  |
| 19 | Blood, fungus, biological agents | x |  | Nitrile gloves | Hazard when workiing in chemical. Nitrile gloves required. |
| 20 | Exposure to live electrical currents | x |  | Electrical gloves; consult HSE Representative | Hazards during troubleshooting live electrical and performing live electrical work. Arc flash protection required. |
| 21 | Sharp tools, machine parts, etc. | x |  | Leather or Kevlar gloves |  |
| 22 | Material handling | x |  | Leather gloves | General material handling activities. General use gloves required. |
| FOOT HAZARDS |  | Yes | No | Recommended PPE | If yes, describe hazard details |
| 23 | Heavy materials (greater than 50 pounds) handled by employees | x |  | Safety shoes or boots | Exposed to heavy material during general maintenance activities. |
| 24 | Potential to crush whole foot |  | x | Safety shoes or boots with metatarsal guard |  |
| 25 | Sharp edges or points (puncture risk) |  | x | Safety shoes or boots |  |
| 26 | Exposure to electrical hazards | x |  | Safety shoes or boots with: Conductive - Protects the wearer in an environment where the accumulation of static electricity on the body is a hazard. Static dissipative - Reduces the accumulation of excess static electricity by conducting body charge to ground while maintaining a sufficiently high level of resistance. Electrical hazard - Provides a secondary source of protection against accidental contact with live electrical circuits, electrically energized conductors, parts or apparatus, and is manufactured with nonconductive electrical shock resistant soles and heals. | Hazards during troubleshooting live electrical and performing live electrical work. Arc flash protection required. |
| 27 | Slippery conditions | x |  | Rubber-soled boots or grips |  |
| 28 | Chemical contamination | x |  | Rubber, PVC, or polyurethane boots or boot covers with puncture and protective toe if task required | Chemical exposure risk in chemical areas during cleanup activities. Rubber over’s required. |
| 29 | Wet conditions |  | x | Rubber boots or boot covers |  |
| 30 | Construction/demolition |  | x | Safety boots with metatarsal guard if foot crushing hazard exists |  |

| FALL HAZARDS |  | Yes | No | Recommended PPE | If yes, describe hazard details |
| --- | --- | --- | --- | --- | --- |
| 31 | Elevations above 4 feet (general industry) or 6 feet (construction) without guardrails | x |  | ANSI A-10.14 Type 1 full-body harness | Work will be common at heights greater than four feet. Work performed in aerial lifts and on machines. Fall protection required. |
| 32 | Suspended scaffolds, boatswain’s chairs, float scaffolds, or suspended staging |  | x | ANSI A-10.14 Type 1 full-body harness |  |
| 33 | Working in trees |  | x | ANSI A-10.14 Type 1 full-body harness |  |
| 34 | Working in vehicle-mounted elevating work platforms (e.g., bucket trucks, aerial lifts) | x |  | ANSI A-10.14 Type 1 full-body harness | Work will be common at heights greater than four feet. Work performed in aerial lifts and on machines. Fall protection required. |
| WATER HAZARDS |  | Yes | No | Recommended PPE | If yes, describe hazard details |
| 35 | Working on or above water where a risk of drowning exists |  | x | U.S. Coast Guard approved personal floatation device; Type I, II, or III |  |
| EXCESSIVE HEAT OR FLAME |  | Yes | No | Recommended PPE | If yes, describe hazard details |
| 36 | Full body chemical protective clothing in temperatures greater than 80 degrees F |  | x | Cooling vest |  |
| 37 | Work around molten metal or flame |  | x | Nomex or heat reflective clothing |  |
| 38 | Welding activities | x |  | Welding leathers for those areas that are exposed to flame, spark, or molten metal | Welding operations. Welding jacket required. |
| RESPIRATORY HAZARDS |  | Yes | No | Recommended PPE | If yes, describe hazard details |
| 39 | Airborne particulates, gases, vapors, or mists in excess of established exposure limits | x |  | Refer to SMS 042 or URS HSE Representative for respirator selection guidance |  |
| EXCESSIVE NOISE |  | Yes | No | Recommended PPE | If yes, describe hazard details |
| 40 | Exposure to noise | x |  | Ear plugs, muffs or both | Hearing protection required throughout the facility. |
| BODY AND LEG PROTECTION |  | Yes | No | Recommended PPE | If yes, describe hazard details |
| ,41 | Chemical exposure | x |  | Contact URS HSE Representative for assistance in proper selection | Chemical exposure risk in chemical when handling chemicals. Chemical protective apron required. |
| 42 | Using chainsaw, cutting brush |  | x | Chainsaw chaps |  |
| 43 | Exposure to snakes | x |  | Snake chaps | Cobras, snake chaps required at all times. |

# Additional input if required.