# WELDING LENS SELECTION

| Operations | Electrode Size (1/32”) | Arc Current | Minimum Protective Shade |
| --- | --- | --- | --- |
| Shielded metal arc welding (SMAW) | Less than 3 | Less than 60 | 7 |
| SMAW | 3 – 5 | 60 – 160 | 8 |
| SMAW | 5 – 8 | 160 – 250 | 10 |
| SMAW | More than 8 | 250 – 550 | 11 |
| Gas metal arc welding and flux cored arc welding |  | Less than 60 | 7 |
| Gas metal arc welding and flux cored arc welding |  | 60 - 160 | 10 |
| Gas metal arc welding and flux cored arc welding |  | 160 – 250 | 10 |
| Gas metal arc welding and flux cored arc welding |  | 250 - 500 | 10 |
| Gas tungsten arc welding |  | Less than 50 | 8 |
| Gas tungsten arc welding |  | 50 – 150 | 8 |
| Gas tungsten arc welding |  | 150 - 500 | 10 |
| Air carbon arc cutting | (light) | Less than 500 | 10 |
| Air carbon arc cutting | (heavy) | 500 – 1000 | 11 |
| Gas tungsten arc welding |  | Less than 20 | 8 |
| Gas tungsten arc welding |  | 20 – 100 | 8 |
| Gas tungsten arc welding |  | 100 – 400 | 10 |
| Gas tungsten arc welding |  | 400 – 800 | 11 |
| Plasma arc cutting | (light) | Less than 300 | 8 |
| Plasma arc cutting | (medium) | 300 – 400 | 9 |
| Plasma arc cutting | (heavy) | 400 -800 | 10 |
| Torch blazing |  |  | 3 |
| Torch soldering |  |  | 2 |
| Carbon arc welding |  |  | 14 |
| Gas welding |  |  | 5 – 6 |
| Oxygen cutting |  |  | 3 - 5 |