# POWERED HAND TOOLS – Part 2

What should you do before using powered hand tools?

- Ensure that you have been properly trained to use the tool safely.  Read the operator's manual before using the tool and operate the tool according to the manufacturer's instructions.

- Ensure that the power tool has the correct guard, shield or other attachment that the manufacturer recommends.

- Prevent shocks.  Ensure that the tools are properly grounded using a three-prong plug, are double-insulated (and are labeled as such), or are powered by a low-voltage isolation transformer: this will protect users from an electrical shock.

| - Check electric tools to ensure that a tool with a 3-prong plug has an approved 3-wire cord and is grounded. The three-prong plug should be plugged in a properly grounded 3-pole outlet. If an adapter must be used to accommodate a two-hole receptacle, the adapter wire must be attached to a known, functioning ground. Never remove the third, grounding prong from a plug. - Replace open front plugs with dead front plugs. Dead front plugs are sealed and present less danger of shock or short circuit. |  |
| --- | --- |

- Have a qualified electrician install a polarized outlet if the polarized, two-prong plug of a double-insulated tool does not fit in a two-hole receptacle.  Double insulated tools use plugs having one prong that is visibly wider than the other.  If the plug does not fit in a receptacle after reversing the plug, the wall receptacle may be an older, non-polarized type.  These can only accommodate plugs with two prongs that are the same width.

- Test all tools for effective grounding with a continuity tester or a ground fault circuit interrupter (GFCI) before use.

- Use only the kind of battery that the tool manufacturer specifies for the battery-powered tool that you are using.

- Recharge a battery-powered tool only with a charger that is specifically intended for the battery in that tool.

- Remove the battery from the tool or ensure that the tool is switched off or locked off before changing accessories, making adjustments, or storing the tool.

- Store a battery pack safely so that no metal parts, nails, screws, wrenches and so on can come in contact with the battery terminals; this could result in shorting the battery and possibly cause sparks, fires or burns.