Slips and Trips Hazard Checklist

This document outlines potential slip and trip hazards in a workplace and potential corrective actions to be considered in addressing these hazards.

This tool is primarily intended to assist joint health and safety committees/health and safety representatives in conducting and recording an inspection with a focus on slip and fall hazards. This specialized type of inspection may be warranted when:

- There are routinely slip and trip injuries at a work location

- There is a recent upward trend in slips, trips and falls at a work location

- There has been a recent very serious slip or trip injury at a work location

- Recent general workplace inspection reports have identified a number of slip and/or trip hazards

[NOTE: This checklist should be used as a supplement to, not a replacement of, a general workplace inspection report. It may be used in whole or in part, depending on the layout of the workplace and work operations].

This tool is also intended to assist management of areas where slip or trip hazards have been identified in:

- Identifying some potential corrective actions to be considered

- Tracking of actions taken in response to identified hazards

- Communicating with joint health and safety committees/health and safety representatives regarding actions taken or planned

| Hazard/Potential Hazard | Present (Y or N) | Potential actions to be taken | Actions to be taken | By whom | By when |
| --- | --- | --- | --- | --- | --- |
| Outdoor areas |  |  |  |  |  |
| Outdoor areas Can anything be found on the paths, steps and fire escapes that could cause slips, e.g. build-up of leaves, wet grass, moss, mud etc? |  | - Set up a regular work schedule for clearing paths, tackle busy routes first - Consider cutting back plants and trees that overlap paths |  |  |  |
| Are paths prone to ice build-up during winter months? |  | - Consider alternative, safer routes - Monitor weather conditions and put a winter procedure in place, e.g. gritting, salting |  |  |  |
| Are there any changes in level on the path that are not easy to see, e.g. small slopes? |  | - Highlight hazard – improve lighting, apply contrasting eye-catching colour to slope (e.g. non-slip paint, flush-fitting bolt-on material) |  |  |  |
| Are there holes, potholes, or uneven paving on footpaths? |  | - Barrier off area as a temporary solution, ensure barriers cannot be easily moved - Highlight hazard, e.g. improve lighting, use eye-catching colour on defective area as a temporary solution - Maintenance required – fill in holes, re-lay paving, replace broken paving stones |  |  |  |
| Are fire escapes slippery when wet? |  | - Improve grip – consider applying slip-resistant coating/strips or bolt on slip-resistant material (caution – do not create a trip hazard) |  |  |  |
| Doorway |  |  |  |  |  |
| Is the floor between the building threshold (entrance) and the entrance matting slippery when wet? |  | - Improve grip – consider extending mat or exterior paving, applying slip-resistant coating/strips or changing to more slip-resistant material |  |  |  |
| Entrances |  |  |  |  |  |
| Is there water on the floor from rain etc? Is it making the floor slippery |  | - Stop water entering building – construct canopies over entrances - Improve external drainage, keep doors closed when you can - Prevent water spreading – fit large and absorbent entrance mats for drying shoes - Remove water quickly – review cleaning system, introduce dry mopping, consider introducing heaters/under floor heating to speed up drying time - Improve grip - Consider fitting slip-resistant flooring |  |  |  |
| Are there any trip hazards in the area, e.g. trailing cables, deliveries, mats with curled up edges, or other objects? |  | - Housekeeping needed – tidy away cables, provide safe delivery storage area, clear away boxes and equipment, fix down mat edges or replace if necessary |  |  |  |
| Corridors and offices |  |  |  |  |  |
| Are there any subtle changes in floor level, e.g. slopes, small steps, abrupt changes from one flooring material to another? |  | - Highlight hazard – improve lighting, use eye-catching colour on slope/step, clearly highlight change from one flooring material to another |  |  |  |
| Are the floors smooth in areas where contamination can be found on the floor (e.g. liquids, food and food wrappers, dusts, polythene, condensation etc?) |  | - Stop contamination from getting onto floor – provide bins for litter, fix leaks, fit lids on containers, close doors leading from working areas - Prevent spreading – drip trays beneath plants/machines/water coolers - Remove contamination quickly – review cleaning system, spot clean spills, dry mop large wet areas, vacuum/brush up dry materials |  |  |  |
| Are the tiles or flooring becoming unstuck or curling at the edges? Are there holes? |  | - Maintenance required – fix down tiles and carpet edges, replace if necessary, fill in holes, replace cracked tiles |  |  |  |
| Is the anti-slip floor coating or grip tape worn smooth or damaged? |  | - Maintenance required – replace damaged and worn coatings - Consider changing flooring |  |  |  |
| Are there any trip hazards around workstations or in corridors and walkways, e.g. trailing cables, boxes, deliveries, equipment or other objects? |  | - Housekeeping needed – keep walkways clear, tidy away or use cable covers, provide additional storage, clear away boxes and equipment |  |  |  |
| Are light levels too low to see the floor surface clearly? |  | - Improve lighting – new bulbs, additional lights |  |  |  |
| Is light reflecting on the smooth flooring creating glare? |  | - Improve lighting – re-angle lights, install blinds or anti-glare grills or glazing films - Consider removing floor surface shine |  |  |  |
| Stairs and ramps |  |  |  |  |  |
| Are step nosings (edge of step) hard to see, rounded, damaged or slippery? |  | - Check lighting is sufficient to see step edges clearly - Highlight the very edge of the step with a nosing that has a high visibility, square edge and non-slip finish - For difficult to replace round-edged nosing, ensure non-slip edging wraps right around the edge of the nosing |  |  |  |
| Are handrails available? Are they easy to reach and useable? |  | - Provide a handrail on at least one side of the stairs; with additional handrails provided for stairs of larger length - Handrail heights should be between 900 mm and 1000 mm and be parallel to the pitch line (slope) of the flight of stairs. - On landings where the handrail provides guarding the height should be 1100 mm - Recommendations for handrail shape, diameter and distance from wall can be found in the Ontario Building Code |  |  |  |
| Is the height (rise) of the steps or depth of tread (going) inconsistent throughout the flight? |  | - Highlight the problem, e.g. with warning notice - Correct the rise/going of the stairs so they are all of equal height |  |  |  |
| Are the stair treads slippery? |  | - Thoroughly clean on a regular basis to remove contaminants - Replace stair covering with one with better slip resistance |  |  |  |
| Are any ramps or slopes in or around the workplace difficult to see? |  | - Highlight ramp with contrasting colour and check lighting levels - Improve grip – consider fitting slip-resistant flooring - As with flights of stairs, consider providing handrails |  |  |  |
| Work areas and work platforms, kitchens, warehouses, garages, storerooms and cold stores |  |  |  |  |  |
| As part of the work process is contamination (fluids, solids, dust, debris etc) getting onto the floor? Is the floor slippery? People – spillages, overfilling containers, clearing waste off work surfaces onto the floor, discarding debris onto the floor Machines – leaks, overspray, spills, by-product Process – overspills, leaks, by-product |  | - Stop contamination from getting onto floor – change system of work, improve work area layout, provide bins, dust extraction, lids on containers, reduce quantity of product in containers, fix leaking machinery - Prevent contamination spreading – use drip trays, screens to stop splashes, good floor drainage, high-lipped sinks - Remove contamination quickly – spot clean spills, dry mop large wet areas, vacuum/brush up dry materials - Improve grip – consider slip-resistant flooring; provide slip-resistant footwear |  |  |  |
| Is condensation forming on the floor? Is condensation forming on overhead pipe-work and dripping onto the floor? Is the floor slippery? |  | - Improve ventilation – use extraction - Insulate overhead pipe-work - Improve grip – consider slip-resistant flooring; provide slip-resistant footwear |  |  |  |
| Is poor drainage causing a pooling of fluids on the floor? |  | - Improve floor drainage |  |  |  |
| Cold store – is there ice build-up on the floor? Is the floor slippery? |  | - Remove ice - Door maintenance – check door closes and seals properly – replace seals, fix door and frame - Prevent humidity, e.g. fit automatic doors, curtains, humidity controls - Consider supplying slip-resistant footwear |  |  |  |
| Are designated walkways unusable or partially blocked? |  | - Create a clear and even walkway through the workplace - Housekeeping needed – tidy away cables, provide additional storage, clear away clutter, boxes and equipment, safely store pallets |  |  |  |
| Are walkways uneven, do they have holes or missing tiles? |  | - Barrier off area as a temporary solution, ensure barriers cannot be - easily moved - Highlight hazard, e.g. improve lighting, use eye-catching colour on - defective area as a temporary solution - Maintenance required – fill in holes, re-lay/replace defective flooring |  |  |  |
| Are there any raised carpet edges or holes? |  | - Firmly stick down raised or loose edges - Maintenance required – replace all or damaged section of carpet |  |  |  |
| Are the tiles or flooring becoming unstuck or curling at the edges? |  | - Firmly stick down loose tiles and raised edges - Maintenance required – replace all or damaged section of flooring |  |  |  |
| Are there any trip hazards around workstations, e.g. trailing cables, boxes, deliveries or other objects? |  | - Encourage a ‘see it, sort it’ mentality among staff - Housekeeping needed – keep walkways clear, tidy away or use cable covers, provide additional storage, clear away boxes and equipment |  |  |  |
| Are light levels too low to see clearly? Is light bouncing off the flooring creating glare? |  | - Improve lighting – new bulbs, re-angle, additional lights, install antiglare grills - Stop glare – consider removing floor surface shine |  |  |  |
| Toilets, bath and shower rooms |  |  |  |  |  |
| Is water getting onto the floor? Is the floor slippery when wet? |  | - Stop water getting onto the floor – improve shower curtains/ screens, position sufficient hand dryers close to sinks - Remove water quickly – regular monitoring, spot clean, dry mop wet areas - Improve floor drainage where possible - Improve grip – consider fitting slip-resistant flooring |  |  |  |
| Are taps or pipes leaking? |  | - Prevent contamination spreading – provide drip trays as a temporary solution - Maintenance required – fix leaks and taps |  |  |  |
| Cleaning |  |  |  |  |  |
| Are spillages left on the floor for some time before they are cleaned up? |  | - Encourage a ‘see it, sort it’ mentality among staff - Ensure spills cleaning equipment is readily available for use - Review/improve cleaning regime and timings of cleaning schedule |  |  |  |
| Are small spills wet mopped? |  | - Spot clean small spills using absorbent cloth/paper towel - Provide training and then supervise - Ensure spills cleaning equipment is readily available for use |  |  |  |
| Can people walk through areas during wet mopping or when floors are still wet? Is the floor smooth or slippery when wet? |  | - Keep people off smooth wet floors – Barrier off/close off areas, wet mop out of hours when no-one is around - Reduce drying time – dry mop the floors with a clean, dry mop |  |  |  |
| Are warning signs left out long after the spill has gone and floor has dried? |  | - Remove cones and signs as soon as cleaning completed and floor is dry - Provide training and then supervise |  |  |  |
| Does the floor look dirty even though it has just been cleaned? |  | - Check manufacturers’/suppliers’ cleaning instructions are being followed - Review floor cleaning method, alter to suit floor type - Provide training on new method and then supervise |  |  |  |
| Are people slipping on the floor even though it has been cleaned and is dry? |  | - Thoroughly clean to remove build-up of polish, grease etc - Review and alter floor cleaning method - Provide training on new method and then supervise |  |  |  |
| Can cleaning equipment leads be seen crossing or blocking walkways, creating a trip hazard? |  | - Coil up unused equipment cable - Change power source – provide additional power sockets; use socket nearest area being cleaned - Consider change to battery-powered equipment - Provide training on new method and then supervise |  |  |  |
| Do bin bags/cleaning equipment in walkways create trip hazards? |  | - Provide training on awareness of trip hazards and how to avoid them and then supervise |  |  |  |
| Tasks |  |  |  |  |  |
| Do tasks stop people seeing slip or trip hazards, e.g. carrying items that restrict view, upset people’s balance, rushing? |  | - Review and improve manual handling and moving procedures - Review work activity |  |  |  |

This checklist was drawn from a document produced by the Health and Safety Executive in the U.K. entitled Slips and Trips Hazard Spotting Checklist.