Two-Wheel Hand Trucks

Two-wheel hand trucks are great for moving just about anything. Whenever furniture, boxes, books or components must be moved, a two-wheel truck really does help reduce the load on the back and shoulders. But, two-hand trucks can produce hazards for the users. One issue with two-wheel trucks is that the load must still be lowered down to the toe plate at ground level. After the load is delivered, it must be lifted again. Review these hand truck safety tips!

| WORKSAFE TIPS | CURBS AND STAIRS |
| --- | --- |
|  |  Employees who deliver to locations without |
| PERSONAL PROTECTIVE EQUIPMENT | docks must pull fully loaded hand trucks |
|  Gloves that increase ability to grip cases. | over uneven walking surfaces, up and over |
|  Footwear with protective toe and midsole. | curbs and up stairways. |
|  |  Provide and use curb ramps. |
| BEST PRACTICES |  Curb ramps allow for easy movement over |
|  For large quantity loads provide a four-wheel | uneven walking surfaces eliminating the |
|  | need to lift and pull fully loaded hand |
| platform-style cart or a two-wheel hand truck |  |
|  | trucks. |
| that converts to a four-wheel platform-style |  |
|  |  Use trucks that have several doors on the |
| truck. |  |
|  | side so loads can be accessed by means |
|  This will minimize bending the torso. |  |
|  | other than the back door. |
|  Use a two-wheel truck with an adjustable toe |  |
|  |  A short ramp from a side door could |
| plate – adjust the toe plate to approximately |  |
|  | eliminate pulling the hand cart up steps and |
| knee height to minimize torso bending. |  |
|  | curbs. |
| INCLINES |  |
|  When pushing up an incline the employee | MAINTENANCE |
| frequently has to lower the angle of the load |  A poorly maintained hand truck will greatly |
| to generate enough force to push the load up | increase physical stress on an employee. |
| the slope. |  |
|  |  When hand truck tires are under or |
|  Lowering the load angle increases the | unevenly pressurized, the employee's arms, |
| amount of weight the employee must lift. In |  |
|  | back, and legs must provide more force to |
| addition to providing force in a horizontal |  |
|  | move the hand truck. |
| direction, the employee must now provide |  |
|  |  Keep balloon tires in good repair and |
| force in a vertical direction. | properly inflated. |
|  This places additional stress on the shoulders, |  |
|  |  Check tire pressure on a quarterly basis or |
| back and legs. |  |
|  | more often if needed. |
|  Provide a fold down third wheel on the back |  |
|  |  Make available to the employees a pressure |
| of the hand truck so the load can be self |  |
|  | gauge, air compressor, and pressure hose. |
| supporting. |  |
|  |  Hard rubber tires do not have the air |
|  This ensures that the employee must only | pressure problems associated with balloon |
| provide force in a horizontal direction. |  |
|  | tires. |
|  |  However, hard rubber tires do not move |
|  | very well over rough terrain. |